Hay Feeder Instructions:

Panel Screws		30	Item A

Nuts			30	Item B

Back Panel		1	Item C

Top				1	Item D

Front Panel		1	Item E

Vertical Panels	5	Panel 1 = Item F

Panel 2-4 = Item G

Panel 5 = Item H

Mounting Frame	1	Item I

4 mounting bolts	= Item J

4 locking washers = Item K

4 flat washes = Item L

4 nuts = Item M

Wood Bolts		3	Item N

Steel Bolts		3	Item O

Front Door		1	Item P

Protective Corner	 for TOP		2	Item Q

Protective Corner for BOTTOM	2	Item R

Also included:

Outdoor electrical box (preinstalled)

Timer (preinstalled)

Battery (preinstalled)

NOTES:	a. Work gloves must be worn to protect from sharp

sheet metal edges

b. Read all directions before you begin

c. Two-person assembly

d. Assemble on work bench

STEP ONE

- Stand panel ONE (F) up vertically with the back panel (C) standing up vertically and INSIDE the panel ONE fold, matching the               marks.

TOP VIEW:

- Using 2 panel screws (A), from the outside/back of the unit, screw into the middle and bottom holes only, using a nut (B) on the inside of the unit.  Tightening by hand at this time:

- Attach Panel TWO (G):

  - The TRAP DOOR must be horizontal, swing door up

  - Match the            marks

  - Insert the PIVOT ROD into the rear bottom hole

  - Using 2 panel screws, from the outside/back of the unit, screw into the middle and bottom holes only, using a nut on the inside of the unit.  Tightening by hand at this time.

- Attach Panel THREE (G):

  - The TRAP DOOR must be horizontal, swing door up

  - Match the               marks

  - Insert PIVOT ROD

  - 2 panel screws into the middle and bottom holes only, using a nut on the inside of the unit. Tightening by hand at this time.

- Attach Panel FOUR (G):

  - The TRAP DOOR must be horizontal, swing door up

  - Match the               marks

  - Insert PIVOT ROD

  - 2 panel screws into the middle and bottom holes only, using a nut on the inside of the unit.  Tightening by hand at this time.

- Attach Panel FIVE (H):

  - The back panel will go inside the fold of panel FIVE

  - The TRAP DOOR must be horizontal, swing door up

  - Match the         marks

  - Insert PIVOT ROD

  - 2 panel screws into the middle and bottom holes only, using a nut on the inside of the unit.  Tightening by hand at this time.

TOP VIEW:

STEP TWO

- Affix the Front Panel (E) using panel screws from the front and nuts inside the unit, aligning the             marks on the Front Panel and panel ONE

FRONT VIEW:

Panel 5

Panel 1

- Hand-tighten front panel screws and nuts on panels 2-5.

STEP THREE

- WIRING

  - ?

  - ?

  - ?

STEP FOUR

Attaching Frame (I)

- Using four 3/8 inch bolts (J), flat washers (K), locking washers (L), and bolts (M), attach the Mounting Frame to the outside of the unit at the     points on the outside of panel 1 and panel 5

STEP FIVE

- Secure TOP (D) using ??? panel screws and nuts, aligning the	  marks

- Hand-tighten screws

STEP SIX

- Using a square, check your work.

- Check the back and front:

STEP SEVEN

TIGHTEN all nuts / bolts, keeping in square

STEP EIGHT

Mounting Frame Prep (Included are 3 bolts for mounting on WOOD [N] and 3 bolts for mounting on STEELE [O])

- Our recommendation is to measure 69 inches from ground to where you will pre-drill the top anchor hole > drill hole.

- MARK the top anchor hole vertically and horizontally so it’s easy to find:

STEP NINE

- It would be a good idea to affix a 2x4 (36 inches long) on the wall to SET THE UNIT ON when putting it up to the wall:

PREDRILLED ANCHOR HOLE

- If you follow the recommended height above for the top anchor hole, place the top of this temporary support 47.25 inches ground up.

Predrilled

Anchor hole

2x4 support

- Also, a temporary 47 inch front support is helpful when holding the unit up on the wall.

- Make sure all trap doors are still locked in place and LIFT UNIT

- Using either the wood mounting screw or the 3 metal mounting screw, affix the UNIT to the wall, using the pre-drilled hole from step eight above

- Make sure the unit is LEVEL

- Use the other two mounting screws (wood versus metal) on either side of frame

- REMOVE all temporary supports

STEP TEN

Front Door (P) … can be set for a right or left open

- Remove the Upper Pivot Rod Bracket (upper right)

Lock screw hole

Mount screw

Rod hole

- Drop the pivot rod WITH WASHER into the hole (bottom right or left)

- Place Upper Rod Bracket over the top pivot rod

- Reattach the Upper Pivot Rod bracket to the unit (right or left)

- Adjust as needed

STEP ELEVEN

Affix protective corners

- Two prong on top front (Q):

- Three prong on bottomfront (R):

STEP TWELVE

- UNBOX BATTERY

- PLUG IN POSITIVE (BLACK) AND NEGATIVE (RED) TERMINALS

- PLUG IN 110V BATTERY CHARGER

- PLUG IN 110V CONTROLLER

STEP THIRTEEN

LOADING THE HAY

- Make sure all trap doors are set horizontal / affixed inside the swing-arm notch:

- With front door open, insert hay flakes at an angle:

BIN A   BIN B   BIN C   BIN D

Note:  You can also place cans of grain in the BINS:

Front door closed: