ULINK2: Debug and Serial Wire Trace

- Flash programming + run-control

- Memory + breakpoint (access while running)

- Serial wire trace capturing up to 1 Mbit/sec (UART mode)

- 10 MHz JTAG/SW clock speed

## Specifications

ULINK2, together with Arm Keil MDK, provides excellent debug capabilities for Cortex-M devices. You can control the processor, set breakpoints, and read/write memory contents, all while the processor is running at full speed.

| Wide target voltage range | 2.7V - 5.5V |
| --- | --- |
| Connectivity | USB 2.0 high-speed connection |
| Target Connectivity | Data trace for Cortex-M up to 1 Mbit/s (UART mode) JTAG clock speed up to 10 MHz Supports Cortex-M devices running at up to 200 MHz Memory read/write up to 28 KBytes/s |
| Power | USB powered (no power supply required) |
| Target Connectors | 10-pin (0.05") - Cortex Debug Connector 20-pin (0.10") - Arm Standard JTAG Connector |
| Device Support | ULINK2 connects to a wide variety of processor targets, including Arm7, Arm9, all Cortex-M, 8051, and C166 devices. |
| Debug Interface Support | JTAG support for Arm7, Arm9, 8051, C166, and all Cortex-M processors. Serial Wire Debug (SWD) support for all Cortex-M processors. Serial Wire Viewer (SWV) support for all Cortex-M processors. |
| Run/stop Debugging Features | Download programs to your target hardware. Examine memory and registers. Single-step through programs and insert multiple breakpoints. Run programs in real-time. Program Flash memory. |
| Features | A ULINK debug adapter allows you to debug, trace and analyze embedded programs running on the target hardware. All ULINK adapters enable you to: - Download programs to your target hardware. - Examine memory and registers. - Single-step through programs and insert multiple breakpoints. - Run programs in real-time. - Program Flash memory. - Connect to a target via JTAG or Serial Wire Debug (SWD). - Debug Arm Cortex-M devices on-the-fly. - Examine trace information from Armv7-M and Armv8-M devices. |