Specifications

The MPS4 board is an FPGA prototyping platform, supporting FPGA implementations of Arm Corstone subsystems, including Cortex-A, Cortex-R, and Cortex-M processors and other Arm IP products. The board features a large AMD Virtex UltraScale+ XCVU13P FPGA with 3870k logic elements. It contains a range of debug connectors and peripheral interfaces to support software development and IP evalulation. The MPS4 board provides:

- High performance high LUT count FPGA

- Multiple memory options

- Multiple I/O expansion options

- Wide range of debug connectors

- Enables device driver and software development support

The expansion ports enables users to develop their own breakout boards for connection to the FPGA implementation, as well as supporting connections to other systems or interfaces. Two Arduino connectors enable a range of peripheral shields to be added from multiple vendors. Pmod and HAT connectors support industry-standard plug-in modules. More complex peripherals or interfaces can be supported through the FMC+ HPC or PCIe interfaces. FPGA Encryption enables the use of Arm FPGA Implementations on the board. A collection of different debug interfaces allows the use of Arm processor debug, testing and proving of Coresight. The ILA connector allows users to debug the FPGA design in real-time using ChipScope or Identify.

| FPGA | AMD Virtex UltraScale+ XCVU13P 3780k logical elements (LE) Arm encrypted FPGA image support |
| --- | --- |
| Memory and storage | 11.8GB internal BRAM 128MB QSPI flash 16GB DDR4 (8GB fitted) microSD card interface |
| PCIe systems | 2 PCIe Gen 3x4 slots 4 USB 3.0 ports 10/100/1000 GbE port |
| Expansion | 2 Arduino shield connectors 4 Pmod connectors HAT GPIO connector VITA FMC+ HPC connector 3 ARC6 24x MGT connectors rated at 10Gbps/20Gbps (Note: TRM specifies theoretical limits of 12.5/25Gbps) Onboard RGB to HDMI 1.4a 1080p supporting I2S audio 6 channel Audio In/Out codec 2 MIPI CSI 1.25-Gbps 2-lane camera connectors MIPI CSI 1.25-Gbps 4-lane camera connector MIPI DSI 1.25-Gbps 2-lane display connector Remote access RPi interface 10 user LEDs and 8 user switches |
| Debug | 10-pin CoreSight debug connector 20-pin CoreSight debug connector and 4-bit trace MIPI 60w debug + 16/32-bit trace Onboard ULINKplus debug High speed USB3.0 16-bit FIFO 4 Virtual UARTS ILA for ChipScope or Identify |
| Simplified configuration | Motherboard Configuration Controller (MCC) and microSD card configuration files |
| Deliverables | MPS4 board 252W 12V PSU USB-C cable Basic test software and FPGA image |