C172 CHECKLIST

Vr – 55 kts          – 64 mph

Vy – 81 kts          – 94 mph

Vx – 60 kts          – 69 mph

Vs – 47 kts          – 55 mph

Vso – 41 kts        – 48mph

Vfe – 85 kts      – 98 mph

Vno – 128 kts     --148 mph

Vne – 160 kts     --185 mph

Best Glide – 65 kts      – 75 mph

Va – 97 kts         – 112 mph (at max gross)

BEFORE STARING ENGINE

- Exterior Pre-flight inspection – COMPLETE

- Hobbs/Tach time -- RECORD

- Seats/ Seatbelts -- ADJUST/ ON

- Circuit Breakers—CHECK

- Radios—OFF

- Transponder—OFF

- Lights—OFF

- Fuel Selector -- BOTH

STARTING ENGINE

- Brakes --HOLD/SET

- Beacon--ON

- Carburetor Heat -- COLD

- Mixture -- RICH

- Primer -- AS REQUIRED

- Throttle -- OPEN ¼ inch

- Master Switch -- ON

- Ignition -- ON BOTH

- Propeller area-- CLEAR

- Ignition -- START (Release when engine starts)

- Throttle -- 1000 RPM

- Oil Pressure -- CHECK

- Mixture -- LEAN FOR TAXI

- Flaps -- UP

PRE-TAXI

- Transponder -- STANDBY & 1200

- Radios -- ON & SET

- Timer -- START

- Weather -- CHECK

- Altimeter & Heading — SET

- Brakes -- CHECK

ENGINE RUN-UP

- Brakes -- HOLD/SET

- Flight Controls -- FREE & CORRECT

- Trim --SET

- Mixture -- RICH

- Throttle -- 1700 RPM

  - Engine Instruments – CHECK

  - Magnetos – CHECK (125 RPM max, 50 RPM differential)

  - Carb Heat – CHECK, THEN OFF

  - Ammeter – CHECK

  - Suction Gauge – CHECK (4.6”-5.4” IF APPLICABLE)

- Throttle -- IDLE

  - Carb Heat – CHECK, THEN OFF

- Throttle -- 1000 RPM

- Flight Instruments – CHECK

- Mixture -- LEAN (IF NOT #1)

BEFORE TAKEOFF

- Cabin Doors -- CLOSED

- Flaps – UP (10o FOR SOFT-FIELD)

- Mixture -- RICH

- Carb Heat -- OFF

- Transponder -- ALT

- Landing & Nav lights -- ON

CRUISE

- Landing Light — OFF (OR AS REQUIRED)

- Power — AS REQUIRED

- Trim — AS REQUIRED

- Mixture — AS REQUIRED

BEFORE LANDING

- AWOS/ATIS — CHECK (ABOUT 10MILES OUT)

- Fuel Selector -- BOTH

- Mixture — RICH

- Landing Light — ON

- Seatbelts – ON

- Carb Heat — ON BEFORE CLOSING THROTTLE

AFTER LANDING

- Flaps — UP

- Carb Heat — OFF

- Landing Light — OFF

- Transponder -- STDBY

ENGINE SHUTDOWN

- Brakes — HOLD/SET

- Transponder — OFF

- Radios — OFF

- Throttle — 1000 RPM

- Mixture — IDLE CUT-OFF

- Ignition — OFF, REMOVE KEY

- Hobbs/Tach time -- RECORD

- Master Switch — OFF