C172 EMERGENCY CHECKLIST

ROUGH ENGINE OPERATION

- Fuel Selector – ON BOTH

- Mixture – FULL RICH

- Carb Heat – APPLY

- Engine – CHECK INSTRUMENTS

- Primer – CHECK LOCKED

- Magnetos – CHECK LEFT, RIGHT AND BOTH

- Land – ASAP

IF TROUBLESHOOTING UNSUCCESFUL, PREPARE FOR COMPLETE LOSS OF POWER

COMPLETE LOSS OF POWER (LOW ALTITUDE)

- Prepare for emergency landing without power

EMERGENCY LANDING WITHOUT POWER

- Best Glide – 65 KTS, ESTABLISH

- Landing Spot – LOCATE

- Radio – DECLARE EMERGENCY

- Flaps – AS REQUIRED

- Mixture – IDLE/CUTOFF

- Fuel Selector – OFF

- Ignition – OFF

- Master Switch – OFF

- Seatbelts -- ON

- Doors – UNLATCH PRIOR TO FINAL APPROACH

COMPLETE LOSS OF POWER (SUFFICIENT ALTITUDE)

- Best Glide – 65 KTS, ESTABLISH

- Fuel Selector – ON BOTH

- Mixture – FULL RICH

- Carb Heat – APPLY

- Engine – CHECK INSTRUMENTS

- Ignition – CHECK ON BOTH

- Starter -- ENGAGE (IF PROPELLER STOPPED)

IF TROUBLESHOOTING UNSUCCESFUL, PREPARE FOR EMERGENCY LANDING

EMERGENCY LANDING WITHOUT POWER

- Landing Spot – LOCATE

- Radio – DECLARE EMERGENCY

- Flaps – AS REQUIRED

- Mixture – IDLE/CUTOFF

- Fuel Selector – OFF

- Ignition – OFF

- Master Switch – OFF

- Seatbelts -- ON

- Doors – UNLATCH PRIOR TO FINAL APPROACH

ENGINE FIRE IN FLIGHT

- Mixture – IDLE/CUTOFF

- Fuel Selector – OFF

- Ignition – OFF

- Master Switch – OFF

- Cabin Heat – CLOSE

- Emergency Descent – NOT OVER 160 KTS

- Emergency Landing Without Power -- EXECUTE

ELECTRICAL FIRE IN FLIGHT

- Master Switch – OFF

- Vents – CLOSE

- Fire -- EXTINGUISH

- All other Switches (Except Ignition) -- OFF

- Fuses/ Circuit Breakers – Check

- Faulty circuit(s) – LEAVE DEACTIVATED

- Master Switch – ON

- All Other Switches – ACTIVATE SUCCESSIVELY

ENSURE FIRE IS FULLY EXTINGUISHED BEFORE OPENING VENTS

EXCESSIVE/ INSUFFICIENT RATE OF CHARGE

- Alternator – CYCLE OFF THEN ON

- Ammeter – CHECK FOR NORMAL OPERATION

IF NORMAL OPERATION RESUMES, CONTINUE FLIGHT WHILE MONITORING.

CONSIDER DISCONTINUING FLIGHT ASAP

IF EXCESSIVE/ INSUFFICIENT RATE OF CHARGE CONDITION PERSISTS

- Alternator – OFF

- Non-essential electric equipment – OFF

- Land -- ASAP

LOW OIL PRESSURE

- Oil Temperature – CHECK

  - If normal – LAND ASAP

IF OIL TEMPERATURE IS HIGH, PREPARE FOR COMPLETE LOSS OF POWER

- Landing Spot – LOCATE

- Throttle – REDUCE TO MINIMUM NECESSARY TO MAKE LANDING SPOT

EXECUTE EMERGENCY LANDING

SPIN RECOVERY

- Power – IDLE

- Ailerons –- NEUTRAL

- Rudder – FULL OPPOSITE OF THE TURN UNTIL ROTATION STOPS

- Elevator – RECOVER FROM THE STALL