AVIATE – Air Worthiness Acronym

Annual Inspection

VOR Check - Test Navigation Equipment

I00 Hr. Inspection

Air Worthiness Directives (AD)

Transponder (VFR/IFR) Inspection Every 24 months

Emergency Location Transmitter (ELT) 91.207 Em Freq. 121.5 – (12 mos, 1hr cumulative use, .50 battery life)

Static System Inspection (IFR) Every 24 months

Minimum Equipment 					Required Documents Onboard

Day VFR 91.205 b					Air Worthiness Certificate

Anti-collision lights – red beacon			Registration Certificate

Tachometer						Radio License (international flights)

Oil Pressure Gauge					Operating Handbook

Magnetic Compass					Weight and Balance

Air Speed Indicator

Temperature Gauge

Oil Temperature Gauge

Fuel Gauges (1 per Tank)

Landing Gear Position

Altimeter

Manifold Pressure Gauge

ELT Emergency Location Transmitter

Seat belts – Lap Belt up to 78’

Shoulder & Lap belt after 78’

Night VFR 91.205 c

Fuses

Landing lights (if for Hire)

Anti-Collision Lights

Position Lights

Source of Alternate Power

IFR 91.205 d

Generator/ Alternator

Radios for navigation

Attitude indicator

Ball

Clock

Altimeter

Rate of turn indicator

Directional Gyro (heading indicator)